Problem 1

Looking at the chain of the robot side with joints tagged by A, B, C, and D

- A  is a Revolute joint with 1 Degree of Freedom (DOF)

- B is a Universal joint with 2 DOF

- C is a Revolute joint with 1 DOF

- D is a spherical joint with 3 DOF

Looking at the chain of the human side with joints tagged from 1 to 8

- 1 is a spherical joint with 3 DOF

- 2 is a spherical joint with 3 DOF

- 3 is a revolute joint with 1 DOF

- 4 is a revolute joint with 1 DOF

- 5 is a revolute joint with 1 DOF

- 6 is a revolute joint with 1 DOF

- 7 is a revolute joint with 1 DOF

Problem 2

The arms are in the special mechanism.

Hence,

- m = 6

- Joints J = 9 (6 spherical joins + 3 revolute joints)

- Links N = 8 (ground included)

- Spherical joints has 3 constraints -> 3 * 6 constraints in total

- Revolute joints has 1 constraints -> 1 * 3 constraints in total

DOF	= m(N – 1 J) + spherical-constraints + revolute-constraints

= 6(8 – 1- 9) + 18 + 3 = 9

Problem 3